Lab Practical #05:

Study the concept of VLAN using packet tracer.

Practical Assignment #05:

- Implement the different network structures in VLAN and VLAN trunking. Also check connectivity between them using ping command or PDU utility.

Steps to Configure VLANs in Cisco Packet Tracer

- Set Up the Network Topology

  - Open Cisco Packet Tracer.

  - Place a 2960 switch on the workspace.

  - Add multiple PCs and connect them to the switch using Copper Straight-Through cables.

- Access the Switch Configuration

  - Click on the switch device.

  - Navigate to the Config tab.

- Add VLANs

  - In the left menu, choose VLAN Database.

  - In the VLAN configuration table:

    - Enter a VLAN ID (e.g., 10) and a VLAN Name (e.g., Sales), then press Add.

    - Enter another VLAN ID (e.g., 20) with a Name (e.g., HR), then press Add.

- Configure Switch Ports for VLANs

  - Select an interface such as FastEthernet0/1 from the left panel.

  - Change the Port Mode to Access.

  - From the VLAN dropdown, assign the correct VLAN ID (e.g., 10).

  - Repeat for other interfaces, assigning each to the correct VLAN.

- Assign IP Addresses to End Devices

  - For each PC: go to Desktop → IP Configuration.

  - Assign IP addresses within the same subnet for devices in the same VLAN.

- Test VLAN Connectivity

  - Use the ping command: PCs in the same VLAN should communicate successfully.

  - PCs in different VLANs will be unable to communicate unless a router-on-a-stick setup or a Layer 3 switch is used.